# Quality Requirements for Voice and Video

- Voice and traditional standard definition video packets must meet these recommended requirements to be an acceptable quality call:

  - Latency (delay) ≤ 150 ms

  - Jitter (variation in delay) ≤ 30 ms

  - Loss ≤ 1%

- These are one way requirements, meaning a packet sent from a phone in HQ has 150ms to reach the phone in the branch, and vice versa

- HD video has stricter requirements

# Congestion

- Congestion causes delay to packets as they wait in the queue

- As the size of the queue changes it causes jitter

- There is a limit to the size of the queue. If a packet arrives when the queue is full the router will drop it

- Voice and video calls (and applications) will be unacceptable quality if they do not meet their delay, jitter and loss requirements

# Classification and Marking

- For a  router or switch to give a particular level of service to a type of traffic, it has to recognise that traffic first

- Common ways to recognise the traffic are by COS (Class of Service) marking, DSCP (Differentiated Service Code Point) marking, an Access Control List, or NBAR (Network Based Application Recognition)

## Layer 2 Marking - CoS Class of Service

- There is a 3 bit field in the Layer 2 802.1q frame header which is used to carry the CoS QoS marking

- A value of 0 – 7 can be set. The default value is 0 which is designated as Best Effort traffic

- CoS 6 and 7 are reserved for network use

- IP phones mark their call signalling traffic as CoS 3 and their voice payload as CoS 5

## Layer 3 Marking - DSCP

- The ToS Type of Service byte in the Layer 3 IP header is used to carry the DSCP QoS marking

- 6 bits are used which gives 64 possible values. The default value is 0 which is designated as Best Effort traffic

- IP phones mark their call signalling traffic as 24 (CS3) and their voice payload as 46 (EF)

- There are standard markings for other traffic types, such as 26 (AF31) for mission critical data, and 34 (AF41) for SD video

## Recognising Traffic with an ACL

- An Access Control List can be used to recognise traffic based on its Layer 3 and Layer 4 information

- For example SSH traffic going to and from the router 10.10.100.10 on TCP port number 22

## Recognising Traffic with NBAR

- NBAR (Network Based Application Recognition) can be used to recognise traffic based on its Layer 3 to Layer 7 information

- Signatures can be downloaded from Cisco and loaded on your router which recognise well known applications

# Congestion Management

- Queuing can be used to manage congestion on routers and switches

- CBWFQ (Class Based Weighted Fair Queuing) gives bandwidth guarantees to specified traffic types

- LLQ (Low Latency Queuing) is CBWFQ with a priority queue

- Traffic in the priority queue is sent before other traffic

# MQC Modular QoS CLI

- Cisco QoS configuration uses the MQC Modular QoS CLI

- It has 3 main sections

- Class Maps define the traffic to take an action on

- Policy Maps take the action on that traffic

- Service Policies apply the policy to an interface

(This config isn’t required on the CCNA)

# Shaping and Policing

- Traffic Shaping and Policing can be used to control traffic rate.

- They both measure the rate of traffic through an interface and take an action if the rate is above a configured limit.

- Traffic shaping buffers any excess traffic so the overall traffic stays within the desired rate limit.

- Traffic policing drops or re-marks excess traffic to enforce the specified rate limit.

- Classification can be used to allow different rates for different traffic types.