# Spanning Tree Protocol

STP is used to prevent Loops in Layer 2 (Switches)

Below is the outcome of an ARP request from PC1 to R1, without STP protocol.

- There will be more broadcast traffic on a production network than a single ARP request

- We now have a broadcast storm

- The network will crash because the amount of looping broadcast traffic will quickly overwhelm the switch’s CPU and bandwidth

How it works

- Spanning Tree is an industry standard protocol and is enabled by default on all vendor’s switches

- Switches send Bridge Protocol Data Units out all ports when they come online. These are used to detect other switches and potential loops

- The switch will not forward traffic out any port until it is certain it is loop free

- When the port first comes online it will be in a Blocking state.

- Spanning Tree will detect if the port forms a potential loop

- If there is no loop the port will transition to Forwarding

- The process can take up to 50 seconds

- The BPDU contains the switch’s Bridge ID which uniquely identifies the switch on the LAN

- The Bridge ID is comprised of the switch’s unique MAC address and an administrator defined Bridge Priority value

- The Bridge Priority can be from 0 – 65535, with 32768 being the default

- A Root Bridge is elected based on the switches’ Bridge ID values

- The switch with the lowest Bridge Priority value is preferred (16384 is better than 49152)

- In the case of a tie the switch with the lowest MAC address will be selected

- The switches build a loop free forwarding path Tree leading back to the Root Bridge

# Root, Designated and Blocking Ports

The easy way to figure out which ports are Root, Designated and

Blocking:

- Determine the Root Bridge first (best Bridge ID)

- All ports on the Root Bridge are Designated Ports(Purple)

- Determine the Root Ports on the other switches (lowest cost to Root Bridge)

- The ports on the other side of those links are Designated Ports

- On the links which are left, one port will be Blocking

- Determine the Blocking Port (highest cost path to Root Bridge or highest Bridge ID)

- The ports on the other side of those links are Designated Ports

# Spanning Tree Versions

IEEE Open Standards:

- 802.1D Spanning Tree Protocol (STP). The original Spanning Tree implementation. Uses one Spanning Tree for all VLANs in the LAN.

- 802.1w Rapid Spanning Tree Protocol (RSTP). Significantly improved convergence time. Uses one Spanning Tree for all VLANs in the LAN.

- 802.1s Multiple Spanning Tree Protocol (MSTP). Enables grouping and mapping VLANs into different spanning tree instances for load balancing.

MSTP Load Balancing Example

- The Access Layer switches have PCs attached in multiple VLANs

- CD1 is made the Root Bridge for VLANs 10 – 19

- Traffic for these VLANs is forwarded on the link to CD1 and blocked on the link to CD2

- CD2 is made the Root Bridge for VLANs 20 – 29

- Traffic for these VLANs is forwarded on the link to CD2 and blocked on the link to CD1

- Two Spanning Tree instances run, one for each group of VLANs

Cisco released enhancements to the open standards.

- Per VLAN Spanning Tree Plus (PVST+): Cisco enhancement to 802.1D. Uses a separate Spanning Tree instance for every VLAN. This is the default on Cisco switches.

- Rapid Per VLAN Spanning Tree Plus (RPVST+): Cisco enhancement to 802.1w RSTP. Significantly improved convergence time over PVST+. Uses a separate Spanning Tree instance for every VLAN.

The Cisco versions do not support grouping multiple VLANs into the same instance

PVST+ and RPVST+ Load Balancing Example

- The Access Layer switches have PCs attached in multiple VLANs

- CD1 is made the Root Bridge for VLANs 10 – 19

- Traffic for these VLANs is forwarded on the link to CD1 and blocked on the link to CD2

- CD2 is made the Root Bridge for VLANs 20 – 29

- Traffic for these VLANs is forwarded on the link to CD2 and blocked on the link to CD1

- Twenty Spanning Tree instances run, one for each VLAN

- PVST+ will assign the Root, Designated or Alternate role to ports

- Alternate Ports are Blocking Ports

# STP Configuration

Acc3#show spanning-tree vlan 1

The Root Bridge Election

- Because Spanning Tree selects paths pointing towards the root bridge, it acts as a centre point of the LAN

- Best practice is to ensure a pair of high-end core switches are selected as the 1st and 2nd most preferred Root Bridge

- You can manipulate the Root Bridge election by setting Bridge priority

- The default value is 32768, with the lowest number being most preferred

- In the case of a tie the switch with the lowest MAC address will be selected

- This is liable to be the oldest switch

Core1(config)#spanning-tree vlan 1 root primary

- Configures the Core1 switch to be the Root Bridge

- This will set a Bridge Priority of 24576

Core2(config)#spanning-tree vlan 1 root secondary

- Configures the Core2 switch to be the next most preferred Root Bridge after Core1

- This will set a Bridge Priority of 28672

# Spanning Tree and HSRP Relationship

- HSRP should be configured to match the Spanning Tree path

- In this example R1 should be given a higher HSRP priority than R2 so that it is selected as the HSRP active router

- This allows traffic from the PCs to take the most direct path to their default gateway

- If R2 was the HSRP active router, traffic would have to transit via an extra device over the CD1>CD2 link

Aligned ‘Active/Active’ HSRP & Spanning Tree

- Vlan 10

R1(config)#interface g0/1.10

R1(config)#encap dot1 vlan 10

R1(config-if)#ip address 10.10.10.2 255.255.255.0

R1(config-if)#no shutdown

R1(config-if)#standby 1 ip 10.10.10.1

R1(config-if)#standby 1 priority 110

R1(config-if)#standby 1 pre-empt

R2(config)#interface g0/1.10

R2(config)#encap dot1 vlan 10

R2(config-if)#ip address 10.10.10.3 255.255.255.0

R2(config-if)#no shutdown

R2(config-if)#standby 1 ip 10.10.10.1

R2(config-if)#standby 1 priority 90

- Vlan 20

R1(config)#interface g0/1.20

R1(config)#encap dot1 vlan 20

R1(config-if)#ip address 10.10.20.2 255.255.255.0

R1(config-if)#no shutdown

R1(config-if)#standby 1 ip 10.10.20.1

R1(config-if)#standby 1 priority 90

R2(config)#interface g0/1.20

R2(config)#encap dot1 vlan 20

R2(config-if)#ip address 10.10.20.3 255.255.255.0

R2(config-if)#no shutdown

R2(config-if)#standby 1 ip 10.10.20.1

R2(config-if)#standby 1 priority 110

R2(config-if)#standby 1 pre-empt

CD1(config)#spanning-tree vlan 10 root primary

CD1(config)#spanning-tree vlan 20 root secondary

CD2(config)#spanning-tree vlan 20 root primary

CD2(config)#spanning-tree vlan 10 root secondary

# Spanning Tree Portfast & BPDU Guard

- It can take up to 50 seconds for Spanning Tree to transition a port to a forwarding state when it becomes active

- A loop cannot be formed on ports where a single end host is plugged in

- You can make the port transition to a forwarding state immediately when it becomes active by disabling Spanning Tree on the port

SW1(config)# interface f0/10

SW1(config-if)# spanning-tree portfast

SW1(config)# spanning-tree portfast default

- If you enable Portfast on a port and then a loop is formed through it, a broadcast storm will result

- This can be caused by users adding devices to the network or changing cabling

- You can enable BPDU Guard on Portfast ports to guard against this happening

- If a BPDU is received the port will be shut down

SW1(config)# interface f0/10

SW1(config-if)# spanning-tree portfast

SW1(config-if)# spanning-tree bpduguard enable

SW1(config)# spanning-tree portfast bpduguard default

- Spanning Tree Root Guard prevents an unintended switch from becoming the root bridge

- If a port where Root Guard is enabled receives BPDU’s that are superior than the current root bridge, it will transition the port to root- inconsistent and not forward any traffic over the port

SW2(config)#interface fa0/2

SW2(config-if)#spanning-tree guard root

# EtherChannel

- Etherchannel groups multiple physical interfaces into a single logical interface

- Spanning Tree sees the EtherChannel as a single interface, so it does not block any ports

- We now get the full 20Gbps bandwidth

- Traffic is load balanced across all the links in the EtherChannel

- If an interface goes down its traffic will fail over to the remaining links

NIC Teaming

NIC Teaming combines multiple physical network cards into a single

logical interface

- EtherChannel is also known as:

  - A Port Channel

  - LAG Link Aggregation

  - A link bundle

- NIC Teaming is also known as:

  - Bonding

  - NIC balancing

  - Link aggregation

# EtherChannel - Protocols

- LACP Link Aggregation Control Protocol:

  - Open standard

  - The switches on both sides negotiate the port channel creation and maintenance

  - This is the preferred method

- PAgP Port Aggregation Protocol:

  - Cisco proprietary.

  - The switches on both sides negotiate the port channel creation and maintenance.

- Static Etherchannel:

  - The switches do not negotiate creation and maintenance but the settings must still match on both sides for the port channel to come up.

  - Use if LACP is not supported on both sides.

All protocols are configured with the channel-group command

- The switches on both sides must have a matching configuration

- The member interfaces must have the same settings on both sides:

- Speed and duplex

- Access or Trunk mode

- Native VLAN and allowed VLANs on trunks

- Access VLAN on access ports

## LACP

- LACP interfaces can be set as either Active or Passive

- If SW1’s interfaces are set as Active and SW2’s as Passive, the port channel will come up

- If both sides are Passive, the port channel will not come up

- If both sides are Active, the port channel will come up

- It is recommended to configure both sides as Active so you don’t have to think about which side is which

SW1(config)#interface range f0/23 - 24

SW1(config-if-range)#channel-group 1 mode active

This creates interface port-channel 1

SW1(config)#interface port-channel 1

SW1(config-if)#switchport mode trunk

Configure the interface settings on the port channel

Configure matching settings on the other switch on the other side of the links:

SW2(config)#interface range f0/23 - 24

SW2(config-if-range)#channel-group 1 mode active

SW2(config)#interface port-channel 1

SW2(config-if)#switchport mode trunk

## PAgP Configuration

- PAgP interfaces can be set as either Desirable or Auto

- If one side is Desirable and the other Auto, the port channel will come up

- If both sides are Auto, the port channel will not come up

- If both sides are Desirable, the port channel will come up

- If you configure both sides as Desirable you don’t have to think about which side is which

SW1(config)#interface range f0/23 - 24

SW1(config-if-range)#channel-group 1 mode desirable

SW1(config)#interface port-channel 1

SW1(config-if)#switchport mode trunk

Configure matching settings on the switch on the other side of the links

## Static Configuration

SW1(config)#interface range f0/23 - 24

SW1(config-if-range)#channel-group 1 mode on

SW1(config)#interface port-channel 1

SW1(config-if)#switchport mode trunk

Configure matching settings on the switch on the other side of the links

Show info about etherchannel

show etherchannel summary

## Example

1st Port Channel – Acc3 to CD1 - LACP

Acc3 Port Channel 1:

Acc3 F0/23 – CD1 F0/23

Acc3 F0/24 – CD1 F0/24

CD1 Port Channel 1:

CD1 F0/23 - Acc3 F0/23

CD1 F0/24 - Acc3 F0/24

2nd Port Channel – Acc3 to CD2 - PAgP

Acc3 Port Channel 2:

Acc3 F0/21 – CD2 F0/21

Acc3 F0/22 – CD2 F0/22

CD2 Port Channel 2:

CD2 F0/21 - Acc3 F0/21

CD2 F0/22 - Acc3 F0/22

3rd Port Channel – Acc4 to CD2 - Static

Acc4 Port Channel 1:

Acc4 F0/23 – CD2 F0/23

Acc4 F0/24 – CD2 F0/24

CD2 Port Channel 1:

CD2 F0/23 – Acc4 F0/23

CD2 F0/24 – Acc4 F0/24

4th Port Channel – Acc4 to CD1 - LACP

Acc4 Port Channel 2:

Acc4 F0/21 – CD1 F0/21

Acc4 F0/22 – CD1 F0/22

CD1 Port Channel 2:

CD1 F0/21 – Acc4 F0/21

CD1 F0/22 – Acc4 F0/22

# Multi-chassis EtherChannel

# Layer 3 Etherchannel

Switch1(config)#interface range GigabitEthernet 1/0/1 - 2

Switch1(config-if-range)#no switchport

Switch1(config-if-range)#channel-group 1 mode | active | auto

| desirable | on | passive

Switch1(config)#interface port-channel 1

Switch1(config-if)#ip address 192.168.0.1 255.255.255.252

Switch1(config-if)#no shutdown