# LAN, WAN and MAN Definitions

- A Local Area Network (LAN) is a network that connects computers and other devices in a relatively small area, typically a single building or a group of buildings.

- A Wide Area Network (WAN) is a geographically distributed network that connects multiple Local Area Networks together.

- A Metropolitan Area Network (MAN) is a network that connects computers and other devices in a geographic area larger than a LAN but smaller than a WAN.

# Private vs VPN Connections – Private Networks

- A private network uses links which are dedicated for an individual organisation.

- Local Area Networks are private networks.

- Wide Area Networks can also use physical links which are dedicated for an individual organisation.

- A Virtual Private Network (VPN) provides a virtual tunnel between private networks across a shared public network such as the Internet.

- Traffic travelling over the tunnel is encrypted and only readable by the authorised users on both sides.

- Users can share data over the tunnel as if they were connected with a dedicated private link.

- VPNs allow an organisation to use the same physical links for connectivity to the Internet and between offices.

- Because they use shared infrastructure, VPN connections are typically less expensive than dedicated physical links.

## Site-to-Site VPN

- Site to Site VPN connections are terminated on a router or firewall in each office.

- Software does not need to be installed on user desktops.

- IPsec is typically used for encryption.

## Remote Access VPN

- Remote Access VPN connections are between a router or firewall in the office and VPN software installed on an individual user’s device.

- The user can access the VPN from anywhere with Internet connectivity.

- They usually use SSL (sometimes IPsec) for encryption.

# WAN Connection Options

- Multiple options are available for connecting geographically dispersed offices together.

- Not all options are available in all locations.

- What is commonly used in one region may be considered legacy in another.

- Different providers may use different terminology. I’ll use the terminology used by Cisco for the CCNA exam

Primary WAN Connectivity Options

- Leased Line

- MPLS Multi Protocol Label Switching

- Satellite

- The service provider will typically provide an SLA (Service Level Agreement) with guarantees for uptime and traffic delay and loss on the link.

- Leased lines and Satellite can be used for connectivity to the Internet, for direct connectivity between offices, and/or connectivity between offices over VPN.

- MPLS uses a shared core infrastructure at the service provider. It can be used for connectivity to the Internet and/or connectivity between offices over VPN.

Optical Fiber

- Optical fiber is more suitable for long distances than copper wire

- It is commonly used for service provider backhaul connections, but can also be offered to their customers

- FTTx services:

  - Fiber to the Home

  - Fiber to the Premises

  - Fiber to the Building

  - Fiber to the Neighborhood

WAN Backup and Small Office Solutions

- Less expensive options often aimed at home user Internet access can be used as Internet VPN WAN backup options in corporate environments

- There will typically be no corporate level SLA with these services

- These can be used as the primary WAN connection method to the corporate network from smaller offices and for home users

  - DSL Digital Subscriber Line

  - Cable

  - Wireless eg 4G

Legacy WAN Connectivity Options

- PSTN Public Switched Telephone Network

- ISDN Integrated Services Digital Network

- Frame Relay

- ATM Asynchronous Transfer Mode

- X.25

# Leased Lines

- A leased line is a dedicated physical connection between two locations.

- It has fixed, reserved bandwidth which is not shared with anyone else.

- The same bandwidth is available in both directions.

- The company may own the cable infrastructure but more commonly it is leased from a service provider for a monthly fee, hence the name ‘leased-line’.

- The first location is typically a corporate office.

- The second location is typically:

  - Another corporate office, providing point to point connectivity between the two offices

  - A data centre that’s connected to the company’s existing Wide Area Network, providing multipoint connectivity between offices

  - A data centre that’s connected to the Internet, providing Internet connectivity, and optionally corporate office connectivity over Internet VPN

Leased Line Benefits and Drawbacks

- Leased lines have fixed, reserved bandwidth which is not shared with anyone else.

- The service provider will typically provide an SLA (Service Level Agreement) with guarantees for uptime and traffic delay and loss on the link.

- Leased lines are typically more expensive than the other options.

- There is usually a longer lead time for installation.

- Copper or fiber Ethernet connectivity options to the CPE (Customer Premises Equipment) are becoming more common than serial leased lines

# Satellite

- Satellite connections share the same characteristics as cabled leased lines

- They are typically expensive and low bandwidth

- They may be the only option in hard to reach areas

# Phone Lines

(Not asked in CCNA)

- T1 and E1 links were also commonly used for connections to the PSTN (Public Switched Telephone Network)

- The analog phone cable to your house is capable of carrying one call

- A T1 digital line is capable of carrying 24 concurrent TDM calls, an E1 can carry 30 calls

- VoIP (Voice over IP) using SIP (Session Initiation Protocol) signalling over Ethernet WAN connections to the Telco are popular today

# MPLS (Multi Protocol Label Switching) VPN

- WAN connectivity can be provided over an MPLS infrastructure, usually operated by a service provider

- Traffic from multiple customers can travel over the provider’s shared MPLS network, so this is a VPN service

- Different levels of SLA for uptime and traffic delay and loss are often available at different price points

- Ethernet connections are typically used to the customer router

- MPLS VPNs provide a full mesh topology by default

## Layer 3 MPLS VPN

- MPLS runs across the providers core on the PE and P routers

- The customer CE routers do not run MPLS

- The customer CE routers peer at Layer 3 with the provider PE routers

- Static routes or a routing protocol runs between the CE and PE

- The PE router looks like another customer router to the customer

- The provider’s core routers are transparent to the customer

- The customer sites are in different IP subnets

## Layer 2 MPLS VPN

- The CE devices do not peer with the PE devices. The entire provider network is transparent to the customer

- The provider network acts like a giant switch

- The customer sites are in the same IP subnet(s)

- This may be required for clustering an application over the WAN

- It can also be useful for migrating hosts during Disaster Recovery

Layer 2 MPLS VPN Terminology

- VPLS (Virtual Private LAN Service): Multipoint Layer 2 VPN

- VPWS (Virtual PseudoWire Service): Point to point Layer 2 VPN

# Topology Options

## Hub and Spoke (Star)

- Advantages: Simplicity, centralised security policy

- Disadvantages: Single point of failure, suboptimal traffic flow

## Redundant Hub and Spoke

- Advantages: Removes single point of failure, centralised security policy

- Disadvantages: Higher cost, suboptimal traffic flow

## Full Mesh

- Advantages: Optimal traffic flow

- Disadvantages: Higher complexity and cost

## Partial Mesh

# Internet Redundancy Options